# CSI Specification- Eurotech™ Wood Entry Doors

# SECTION 08 14 66 – EUROTECH™ WOOD VENEER ENTRY DOORS

Areas Highlighted require editing

## PART 1 – GENERAL

### 1.1 SUMMARY

- Related Documents: Drawings, general provisions of the Contract, including General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.

- Section Includes:
    1. Factory-assembled EuroTech™ entry doors with engineered thermal-core construction.
    2. Optional sidelights and transoms.
    3. Factory prefitting, premachining, and finishing.
    4. Multipoint locking system and hardware preparation.

- Related Sections:
    1. Section 06 00 00 – Wood, Plastics, and Composites
    2. Section 07 92 00 – Joint Sealants
    3. Section 08 71 00 – Door Hardware
    4. Section 08 80 00 – Glazing

### 1.2 REFERENCES

- ASTM E283 – Air Leakage of Exterior Windows and Doors

- ASTM E330 – Structural Performance of Exterior Windows, Doors, Skylights

- ASTM E547 / E331 – Water Penetration Resistance

- ASTM E1886 / E1996 – Impact and Cyclic Pressure (for hurricane-rated assemblies)

- WDMA I.S.6 – Architectural Wood Flush Doors

- NFPA 80 – Fire Doors and Other Opening Protectives

- ADA Standards for Accessible Design

- Local building codes enforced by the Authority Having Jurisdiction (AHJ)

### 1.3 SUBMITTALS

- Product Data: Manufacturer’s data sheets including construction details, finishes, core materials, hardware prep, and performance testing.

- Shop Drawings: Indicate door sizes, handing, glazing, hardware locations, and elevations.

- Samples:
    1. Veneer sample – minimum 3" x 4" showing specified species and finish.
    2. Hardware finish sample if requested.

- Warranty Documentation.

- Certification Submittals: Evidence of compliance with applicable performance ratings (hurricane).

### 1.4 QUALITY ASSURANCE

- Manufacturer:
    1. Manufacturer shall have a minimum of ten years’ experience producing engineered wood entry doors of comparable type.
    2. Doors shall be fabricated entirely by a single manufacturer to ensure uniformity in appearance and performance.

- Installer: Minimum two years’ experience installing comparable exterior door systems.

- Performance Requirements:
    1. Design Pressure Rating: +/- 60psf

1.5 DELIVERY, STORAGE, AND HANDLING

- Store doors flat on level surface in clean, dry, and ventilated location.

- Protect from moisture, direct sunlight, and physical damage.

- Do not subject doors to extreme humidity or temperature fluctuations prior to installation.

### 1.6 PROJECT CONDITIONS

- Maintain environmental conditions at 55-65°F and 40-50% humidity during installation.

- Verify rough openings are correct and frames are plumb and square before installation.

### 1.7 WARRANTY

- Manufacturer’s Limited Warranty:
    1. Structural integrity – 5 years.
    2. Veneer delamination and finish –5 years. 
    3. Hardware and multipoint system – per manufacturer warranty.
    4. Hurricane-rated units: 5-year performance warranty.

## PART 2 – PRODUCTS

### 2.1 MANUFACTURER

- Acceptable Brand Manufacturers:

    - Glenview Doors (a brand of BMD Manufacturing)

    - Approved equal products, subject to compliance with the design and performance of this specification and as approved by owner and architect in accordance with Section 01600.

### DOOR CONSTRUCTION

- Type: Glenview EuroTech™ Entry Door System.

- Construction:
    1. Engineered insulated core: Thermal foam insulation bonded to laminated LVL stiles and rails for stability and energy efficiency.
    2. Exterior and interior veneers: Real wood veneer (e.g., European White Oak, Mahogany, Walnut) bonded to exterior-grade substrates.
    3. Edge banding: Solid wood edge matching face veneer for trimming and screw-holding strength.
    4. Thickness: 2-3/4" standard (other sizes available).
    5. Factory-sealed on all six sides prior to shipment.

- Veneer and Finish Options:
    1. Veneer species and grain: Horizontal or vertical grain orientation.
    2. Factory-applied finishes: UV-cured, multi-layer system.
    3. Color and sheen: As selected by Architect from manufacturer’s standard or custom options.

- Core Options:
    1. Standard thermal core (foam with LVL substructure)

- Hardware Preparation:
    1. Factory machined for selected locking system.
    2. Multipoint lock compatible with day/night function.
    3. FritsJurgens pivot hinge preparation for pivot doors as specified.

- Glazing:
    1. Double insulated glass units (IGUs) with argon fill.
    2. Available in clear, clear beveled, satinato, satinato beveled, cathedral, chinchilla, rain, waterfall, seedy baroque reed, or custom glass options.
    3. Glazing stops integral to door design.

### 2.3 PERFORMANCE RATINGS

- Hurricane-rated assemblies available up to ±60 psf, compliant with ASTM E1886/E1996.

- Air infiltration and water penetration tested to ASTM standards.

### 2.4 FACTORY PREFITTING AND PREMACHINING

  - Doors shall be factory-prefit, beveled, and machined per hardware templates.

  - Prefit tolerance ±1/16".

  - Coordinate all hardware with Section 08 71 00.

## PART 3 – EXECUTION

### 3.1 EXAMINATION

  - Verify frames and rough openings are correct, plumb, and square.

  - Inspect door units for damage or manufacturing defects before installation.

  - Do not install damaged or warped doors.

### 3.2 INSTALLATION

  - Install doors per manufacturer’s printed instructions.

  - Seal edges and cutouts immediately after machining.

  - Use only compatible sealants and hardware.

  - Adjust for proper clearances and smooth operation.

  - Install weatherstripping and threshold per manufacturer details.

### 3.3 ADJUSTING AND CLEANING

  - Adjust doors for smooth operation and secure latching.

  - Clean finished surfaces; repair minor damage per manufacturer’s touch-up recommendations.

  - Protect installed doors from construction damage until Substantial Completion.

### 3.4 PROTECTION

  - Protect doors from dirt, moisture, and abuse during remaining construction.

  - Replace damaged doors and components prior to project closeout.

END OF SECTION 08 14 66